# Chandler wobble of Earth and Mars 2021 and step by step notes on Orbital Mechanics and Spacecraft Dynamics

The Earth has a wobble called the Chandler wobble about its rotational axis. It is caused by the nonrigidity of planets. Isaac Newton estimated the value for Earth to within the proper order of magnitude, being off by roughly a factor of 2: 10 kilometers versus approximately 21 kilometers in reality. The value for Mars, much smaller than for Earth, was announced in the literature in October 2020 and online on January 2021, having required nearly 20 years of observation from the perturbations of the orbits of various Martian orbiters. The Chandler wobble, which tells us things about the interior of planets, is the topic of the first chapter.

The Chandler wobble, evidently, tells us that nonrigid rotating spherical bodies deform into oblate speres. Oblateness perturbs the gravitational potential (and thus the gravitational field) from the case of perfect sphericity, of which the shell theorem, also known to Newton, tells us that we may replace with an equivalent point source (or monopole) mass. In the nineteenth century, attempts were made to relate the observed precession of the perihelion of Mercury to the oblateness of the rotating sun, its rotation evident from the motion of sun spots. It turned out that the perturbative effect from the oblateness of the rotating sun was too weak, with the full explanation for the precession of the perihelion of Mercury explained after the development of the General Theory of Relativity by Albert Einstein around 1915.

The maintenance of desired orbits by artificial satellites, evidently, require fuel to overcome the perturbation from planetary oblateness and from other perturbative sources, e.g., mountain ranges, that may be represented in terms of expansions in spherical harmonics. The perturbing effects of the oblateness of stars and planets on their respective satellites, as well as the stability of orbits from various types of perturbation, are the topics of the second chapter: Notes on Orbital Mechanics.

Scattered throughout are practical applications: the stability of spin-stabilized satellites to small perturbations from bits of space debris, and methods for attaining gravitational boosts or reductions in velocity for navigating around or out of the solar system.

A. Alaniz

28 January 2021

# Chapter 1 – Chandler Wobble

We will derive equations to explain the Chandler wobble for Earth, and recently for Mars.

## Euler equations of motion (Baierlein 1983)

R

rIα

rα

e3

e1

e2

mα

m3

m2

m1

We begin the investigation of the Chandler wobble with a review of extended, rigid bodies in rotation (figure below).

ω

e3

We have rIα=R+rα where rα=xiαeit, and the xiα are constants. The velocity for mα is given by

vIα=R+dxiαdtei+xiαdeidt =R+0+xIαω×ei=R+0+ω×xIαei=R+ω×rα.

deidt follows from considering the figure below for the three unit direction vectors.

Newton’s second law tells us that the force is given by the time rate of change of momentum. Hence

αddt mαvIα=ddt αmαR+ω×αmαrα=αFα.

If there are many particles, the center of mass, RCM is given by

RCM=1MαmαrIα.

### Equation of location

If we let R=RCM,  and

αFα=ddtMtotRCM

since αmαrIα=0. The displacement of an extended, rigid body’s center of mass is produced from external forces. Internal forces, maintaining the shape of the body (keeping fixed the relative separations between points in the body) cancel out from Newton’s third law (equal and opposite reaction).

### Equation of orientation

Torques often change the orientation of an extended, rigid body. The net torque is given by

αrα×ddtmαvIα=αrα×Fα.

The argument of the left-hand-side may be expressed as:

αrα×ddtmαvIα=ddtαrα×mαvIα-αdrαdt×mαvIα.

The angular momentum is defined by

L=αrα×mαvIα.

The second term can be made to vanish in one of two ways:

αdrαdt×mαvIα=αmαω×rα×R+ω×rα=αmαω×rα×R.

If we chose the center of mass of the extended, rigid body to be the origin, αmαrIα=0, or if the body has a point spatially fixed in the inertial frame of reference and we place the origin of the body frame there, so that R=0, the only term that survives is the time rate of change of the angular momentum:

ddtL=αrα×Fα.

## Moment of Inertia Tensor

L=αmαrα×vIα=αmαrα×ω×rα.

In some particular right-handed orthogonal coordinate system with unit vectors: i, j, k., the above vector triple product leads to L=Lxi+Lyj+Lzk. In this coordinate system, ω=ωxi+ωyj+ωzk. For every particle, rα=rxαi+ryαj+rzαk. Doing the algebra of the above vector triple product leads to

L=IxxIxyIxzIyxIyyIyzIzxIzyIzz=Iω†,

where I is the so-called moment of inertia tensor, which is symmetric, e.g., Ixy=Iyx. Specifically,

Ixx=αmαryα2+rzα2,         Iyy=αmαrxα2+rzα2,  Izz=αmαrxα2+ryα2
Ixy=Iyz=-αmαrxαryα
Iyz=Izy=-αmαryαrzα
Ixz=Izx=-αmαrxαrzα.

The first three are “moments of inertia”. The latter three are “products of inertia”.  In this notation,

Lx=Ixxωx+Ixyωy+Ixzωz
Ly=Iyxωx+Iyyωy+Iyzωz
Lz=Izxωx+Izyωy+Izzωz.

Using Einstein summation convention, torque is now succinctly expressed as

τ=12 ω∙L=12 Iiiωii2+2Ixyωxωy+2Ixzωxωz+2Iyzωyωz.

### Kronecker delta notation

Recall the so-called BAC-CAB rule for triple vector products:

r×ω×r=r∙rω-r∙ωr
=r2ωiei-xjωjxiei
=r2δij-xixjωjei.

We then have: Iij=αmαr2δij-xixj.

#### Example: Ring of radius R

For a ring with symmetry axis along the ‘3’ or z-axis (with vanishing thickness (zα≈0)) we have

I33=αmαr2δ33-z2=αmαxα2+yα2=MringR2.

Off diagonal elements (off diagonals) are zero-valued by symmetry, e.g., I12=-αmαxαyα=0.

I11+I22=αmαxα2+yα2+2zα 2≈αmαxα2+yα2+0=MringR2.

Since I11=I22, I11=I22=12MringR2.

#### Example: Sphere of radius R

I11+I22+I33sphere=spheremα3rα2-xα2+yα2+zα2=spheremα2rα2.

We convert the final sum to an integral with mass density function ρr assumed to be constant:

0R2r2ρr4πr2dr=3∙25MsphereR2.

Since I11=I22=I33, we have Iii=25MsphereR2.

#### Example: Center of an ellipsoid

I1=15 Mb2+c2,  I2=15Ma2+c2,  I3=15Ma2+b2.

For an oblate sphere, we have a=b. If c is only slightly larger than a, a+c≈2a, and a2+c2≈2a2.

### Principle Axes of Inertia

If the inertia tensor (the 3×3 matrix) is diagonalizable (with eigenvectors and corresponding eigenvalues) it does so along axes of symmetry. An axis of symmetry is a principle axis and conversely. Assume I is diagonalizable such that detI=0. This means doing the math of finding the eigenvalues:

Ixx-IIxyIxzIyxIyy-IIyzIzxIzyIzz-I=0

where I∈I1, I2, I3, are the three possible eigenvalues, which need not be distinctly valued. We then find the three possible eigenvectors E=e1, e2, e3. Checking that E E†=I, the identity matrix,  diagonalization proceeds according to the principle axis theorem: E  I E†=Idiag=Id.

In the diagonal basis (with I=I1+I2+I3 in the principle axis):

L=Idω
L=Iiωi
τ=12ω∙L=12Iiωi2.

## Euler’s Equations

Euler's rotation equations (vectorial quasilinear first-order ordinary differential equations) describe the rotation of a rigid body, using a rotating reference frame with axes fixed to the body and parallel to the principal axes of inertia. Recall that net torque equals the time rate of change of angular momentum

dLdt=ddtIijωei =αrα×Fα.

From the middle term, we get

ddtIijωei=Iijωjei+Iijωjdeidt
=Iijωjei+Iijωjω×ei.
=Iijωjei+ω×eiIijωj.

In the principle axes, Iij is diagonal, and the first term leads to

Iijωjei=I11ω1e1+I22ω2e2+I33ω3e3.

The second terms leads to:

ω3e2-ω2e3I11ω1+-ω3e1-ω1e3I22ω2+ω2e1-ω1e2I33ω3.

Collecting terms, we get Euler’s equations:

I11ω1-I22-I33ω2ω3=e1∙αrα×Fα
I22ω2-I33-I11ω3ω1=e2∙αrα×Fα
I33ω3-I11-I22ω1ω2=e3∙αrα×Fα.

## Chandler wobble for Earth (and Mars recently (2020 and 2021)) – (Spiegel 1967)

With the polar axis defined along the z direction, the Earth is approximately ellipsoidal (actually more of an oblate sphere since the principle equatorial diameters along the x and y axes are equal), with approximately 7,900 miles between the north and south poles, and approximately 7,926 miles for the equatorial diameter. See Appendix 1 Chapter 1 for Newton’s estimate of the bulge, about 10 kilometers! (Appendix 2 Chapter 1 extends the treatment above to include the oblateness of planets.)

### Euler equations for planets (and Frisbees)

Since we work on the principle axes, let Iii=Ii (why carry an unnecessary index?). With I1=I2, and noting that we ignore tidal forces and assume zero-valued external torques, we have

I1ω1+I3-I1ω2ω3=0
I2ω2+I1-I3ω3ω1=0
I3ω3=0.

Clearly, ω3=constant=A. Thus we get

I1ω1+I3-I1Aω2=0
I2ω2+I1-I3Aω1=0

Divide both equations by I1, differentiate ω2, and substitute ω1 from the first equation to get:

ω2+I3-I1I1A2ω2=0.

Let

κ=I3-I1I1A.

The general solution for ω2 is ω2=Bcosκt +Csinκt.  At t=0, set ω2=0. Thusly, B=0. Thus ω2=Csinκt, and from I2ω2+I1-I3Aω1=0, we get ω1=Ccosκt. Thus the angular velocity is

ω=ω1e1+ω2e2+ω3e3=Ccosκt e1+Csinκt e2+Ae3.

The angular velocity is constant, equaling ω=ω=C2+A2, precessing around the 3-axis with frequency

f=κ2π=I3-I12πI1A.

If we approximate a planet as a sphere with a ring of mass going around its equator, (Mring≪Mplanet), we may add the moments of inertia of a sphere to those of a ring to get:

Isum=25MsphereR2100010001+MringR2R212000120001MsphereMsphere25  
=25MsphereR21+H0001+H0001+2H

where

H=54MringMsphere.

Thus

f=Ω=I3-I12πI1A=H1+Hω3.

Satellite data indicates MringMsphere=2.17×10-3.  Hence Ω≈1368ω=368 days from west to east. The observed value is nearer 427 days. If the North Pole pokes the Earth along e3, then ω pokes out about 10 meters away.

#### Earth

The Chandler wobble of Earth should have been damped away from dissipative processes of ocean flows and deformation of Earth due to its elasticity. What energy rejuvenates the Chandler wobble is poorly understood. Suggestions include earthquakes, as well as changes in pressure at the bottom of the oceans caused by fluctuations in salinity, temperature, and ocean circulation. Literature from 2016 states:  The wobble may thus run like a heat engine, extracting energy from heat-powered geophysical circulations whose natural periods would otherwise be unrelated to the wobble’s observed period of about fourteen months. This can explain, more plausibly than previous models based on stochastic perturbations or forced resonance, how the wobble is maintained against viscous dissipation (Jenkins 2016).

#### Mars

Scientists have detected the Chandler wobble on Mars, a repeated movement of the poles on the surface of the planet away from its average axis of rotation (Konopliv et al 2020, Lee 2021). Knowledge about the Chandler wobble provides a peek into the interior of Mars: The motion is driven by properties of the mantle, the layer beneath the planet’s crust. The observed period is 206.9 ± 0.5 days.

#### Frisbee with drag proportional to rotational speeds

Given N=-cω, where c is a drag coefficient and N is the external torque from air resistance, for a spinning Frisbee, I3ω3=-cω3=N3. Thus ω3t=ω30e-cI3t. Adding

ω1I1ω1+I3-I1ω2ω3=N1ω1=-cω12
ω2I2ω2+I1-I3ω3ω1=N2ω2=-cω22

leads to Iω1ω1+ω2ω2=-cω12+ω22. Noting that ddtω12=2ω1ω1 and ditto for ω22, we get, with I=I1=I2,

12Iddt ω12+ω22=-cω12+ω22.

Thus

ω12+ω22=e-2cIt.

Trigonometry tells is:

tanθ=ω12+ω22ω3=ecI3tecIt 1ω3=1ω30ec1I3-1It.

If I3>I, then

1I3-1I<0

leading to exponential decay as t→∞.

### Torque and Angular momentum need not be aligned (necessary torques)

The angular momentum vector need not be parallel to the rotation vector. Consider the rotating disc of mass, m, figure below, rotating with constant angular velocity vector ω:

Since ω is constant, ω=0. Euler’s equations reduce to:

-I2-I3ω2ω3=τ1
-I3-I1ω3ω1=τ2
-I1-I2ω1ω2=τ3.

where the τi are the constant torques supplied by the axle to the rotating disc. Note, due to symmetry, that I1=I2=14ma2, while I3=12ma2. Thus τ3=0. Meanwhile:

τ2=-14ma2ω3ω1=-14ma2ω∙ωsinθ=-14ma2ω2sinθ
τ1=-14ma2ω2ω1=-14ma2ω∙ωcosθ=-14ma2ω2cosθ

The angular momentum is L=-τ2j. The torque is τ=τ1e1+τ2e2. They are not aligned.

## Impulses on extended, rigid bodies

Impulse is defined as

I=Fdt.

With zero-valued initial angular velocities, we determine final angular velocities when an angular impulse is applied over time span Δt, say from a BB gun strike at a vector-valued location. Angular impulse is defined as

r×I=r×Fdt=Ndt=I Δω=ΔL.

With Δωi=ωifinal-ωiinitial=ωifinal-0=ωifinal,

I1Δω1=τ1Δt=ΔL1,  I2Δω2=τ2Δt=ΔL2,  I3Δω3=τ3Δt=ΔL3.

Given the impulse in vector form and the vector-valued location of the impact, the angular impulses are determined from:

r×I=ijkrxryrzIimpxIimpyIimpz=ΔL=ΔL1ΔL2ΔL3.

The Δωi equal the ΔLi divided by Ii, the respective principle moments of inertia. If, instead, a spinning satellite is struck and we observe ωt=ω0+Δωt, we proceed by linearization of Euler’s equations.

## Stability of spinning spacecraft

A small angular impulse from space debris striking a stationary satellite leads to it acquiring an angular velocity, changing its attitude. The change in orientation can be corrected if the satellite has thrusters, but the less the thrusters need to be used, the better, since the amount of rocket fuel on board is typically limited (though as of 2020, satellites will start becoming refuellable). A spin stabilized satellite about a principal axis, say the z-axis, may or may not be stable under small perturbations treatable by linearization of Euler’s equations.

Consider a z-axis spinning satellite with initial conditions given below, experiencing no external torques

ω0=ω10ω20ω30=00η

ω10=-I3-I2I1ω20=0ω30=η=0
ω20=-I1-I3I2ω10=0ω30=η=0
ω30=-I2-I1I3ω10=0ω20=0=0.

This is clearly a stable situation before any impact: ω1t=0, ω1t=0, ω1t=0. The initial angular velocities remain unchanging.

Consider a perturbation (small space debris striking the satellite) such that onboard telemetry tells us ωt=ω0+Δωt. Then (with Taylor expansion up to first order: ωt=ω0+Δωt), we have

Δω0t=ωt-0=-I3-I2I1ω2tω3t-I1-I3I2ω1tω3t-I2-I1I3ω1tω2t
=-I3-I2I1ω2t+Δω2tω3t+Δω3t-I1-I3I2ω1t+Δω1tω3t+Δω3t-I2-I1I3ω2t+Δω2tω3t+Δω3t
=-I3-I2I1Δω2tη+Δω3t-I1-I3I2Δω1tη+Δω3t-I2-I1I3Δω2tΔω3t.

With Δω small, terms like Δω2Δω3 are negligible, and are dropped. This linearizes Euler’s equations:

Δω1tΔω2tΔω3t=-I3-I2I1Δω2tη-I1-I3I2Δω1tη0.

Since Δω3=0, Δω3t=constant, leaving a system of two coupled first-order differential equations

Δω1t=-I3-I2I1Δω2tη,  Δω2t=-I1-I3I2Δω1tη.

Differentiate the first to get:

Δω1t=-I3-I2I1Δω2tη
=-I3-I2I1-I1-I3I2Δω1tη2
=I3-I2I1-I3I1I2η2Δω1t=kΔω1t.

The differential equation is like Hook’s law if k is negative, corresponding to a stable oscillatory effect. With k positive, runaway destabilization occurs from small angular impulse perturbations (Ditto for Δω2t). If I3>I1 and I3>I2, k<0. Ditto if I3<I1 and I3<I2. If I3 is less than one of the others, but greater than the remaining one, k>0, we say I3 is the intermediate principle axis (Karnap 2013).

## Appendix 1 Chapter 1

### Isaac Newton’s approximation of the bulge of Earth

Isaac Newton imagined two tunnels from the center of the Earth, filled with ocean water. Since all oceans are in contact with each other, the surface pressure is the same for all the waters. One tunnel runs from the center of the Earth to the North Pole (θ=0), the other from the center of the Earth to the equator (θ=π2). If we express the gravitational acceleration on the surface of the Earth as a function of the radius from the center of the Earth to its surface, and as a function of the polar angle, gr,θ, (and assume the density of the Earth is a constant, ρ), then there will be a difference between g at the North Pole and g at the equator due to the rotation of the Earth itself (ignoring for now any  bulge):
gr,π2≈gr,0-ω2r.

Yet, from the surface pressure of the oceans’ surfaces, the two values of g must be equal:

ΔPr=ρ0RPgr,0dr=ρ0Reqgr,θ2dr.

Then

0RPgr,0dr=0Reqgr,θ2dr
                            =0Rpgr,0dr+RP Reqgr,0dr-0Reqω2rdr.

Hence,

gR,0ΔR≈12 ω2R.

With R=6.38×103 kilometers, the value of ΔR is about 10 kilometers, too small by a factor of 2. Regardless, Isaac Newton knew that the Earth was oblate, and by about how much. Appendix 2 Chapter 1 immediately below extends the treatment above to include the oblateness of planets.

## Appendix 2 Chapter 1

From the first figure of Chapter 1, we studied

vIα=R+dxiαdtei+xiαdeidt.

Since the body in that treatment is rigid, dxIαdt=0. Now consider yourself on a rotating frame, e.g., standing in the middle of a spinning merry-go-round. If you throw a ball radially outwards, it will appear to curve away from the direction of your rotation. For a creature born and raised on the spinning merry-go-round, the laws of physics would look different than to those raised on the approximately flat patch of Earth (and approximate inertial frame) of most people’s lives, where Newton’s laws apply. In transforming Newton’s laws from our relatively good inertial frame to the rotating coordinate system of the merry-go-round, we would pick up so-called fictious forces that the creature of the merry-go-round would take as real, e.g., there is a force that acts perpendicular to the direction of rotation on objects tossed out on radials by the creature. As we have zoomed out from our local neighborhood into space, however, we have realize that Earth itself is not a very good inertial frame on large scales. Astronauts in orbit watching long range artillery shells being fired on the surface of the Earth see the Earth rotating beneath the projectiles while they fly on their ballistic trajectories. Ground-based artillery operators during World War I corrected for these fictious force effects firing shells at Paris from 76 miles away. This particular fictious force is the Coriolis force, which also drives circulation in weather patterns on Earth.

Without going into the full details, velocity and acceleration from a rotating frame of reference become:

rI=R+v+ω×r
rI=R+xiei+ω×v+dωdt×r+ω×v+ω×r
=R+a+ω×v+dωdt×r+ω×v+ω×r

where the index, I, stands for the inertial frame. In the inertial frame,

F=md2ridt.

Hence, from the rotating frame, we have:

a=Fm-ω×v-dωdt×r-ω×v+ω×r.

In Chapter 2, we treat the potential of an oblate planet as the sum of a sphere with a ring about its equator, which we then connect to the quadrupole moment in a spherical harmonic expansion of a lumpy planet. That is, for an object of mass, m, on the surface of the Earth (not in orbit with effective mass, μ):

Veffr=L22μr2-GMsunmplanetr-GMringmplanetR24r3→-12mω2r2sin2θ-GMplanetmr1+MringMplanetR24r21-3cos2θ.

Following Newton’s method of Chapter 1 Appendix 1, of matching geopotentials (per unit mass; m divided out) leads to

-GMplanetRpole1+MringMplanetR2r214-2=-GMplanetRpole+ΔR1+MringMplanetR2r214-ω2R22.

By expansion method of the right:

1Rpole+ΔR=1+ΔRRpole-1≈1-ΔRR

we get that:

ΔRR≈ω2R2GMplanetR2+34MringMplanet≈ω2R2gsurface+34MringMplanet.

With the figure below where the sphere has radius, R and ΔR=a-R:

we can estimate:

MringMplanet=2.17×10-3satellite data≈2πR∙Width∙ΔRρs4π3R3ρ=2πR∙R∙ΔRρs4π3R3ρ=32ρsρΔRR=#ΔRR

where ρ is the average density and ρs is the surface density of the bulge. Thus

ΔRR≈ω2R2g1-34#.

Hence

ΔRR≈ω2R2gsurface+34MringMplanet≈1.73×10-3+1.62×10-3=3.35×10-3,

or Requator-Rpole≈21.4 kilometers. Mount Everest is only 8.8 km above sea!

## References Chapter 1

R. Baierlein, Newtonian Dynamics, McGraw-Hill Book Company, 1983

A. Jenkins, Chandler wobble: Stochastic and deterministic dynamics, arXiv:1506.02810 [physics.geo-ph], 2016

D. Karnap, Dynamics, Stability and Control, 2nd ed., CRC Press, 2013

Alex S. Konopliv, Ryan S. Park, Attilio Rivoldini, Rose‐Marie Baland, Sebastien Le Maistre, Tim Van Hoolst  Marie Yseboodt, and Veronique Dehant, Detection of the Chandler Wobble of Mars From Orbiting Spacecraft, Geophysical Research Letters, 13 October 2020

B. Lee, First Detection of a Built-In Wobble on Another Planet, EOS, Science News by AGU, January 2021

M. R. Spiegel, Theoretical Mechanics, Schaum’s Outline Series, 1967 (19th printing 1992)

# Notes on Orbital Mechanics

These step-by-step notes collect material on orbital mechanics, with an emphasis on the study of the stability of power law potentials and linear combinations thereof through perturbation theory. Contact is made with analog quantum mechanical systems, e.g., the hydrogen atom as described by the Schrödinger equation. This material should lead to an appreciation of navigation in the solar system through gravitational boosts or decelerations.

A. Alaniz, PhD

10 January 2021

# Chapter 2 – Oblate sun – orbital perturbation from the sun as a sphere with an equatorial bulge versus general relativity

Treat the sun as a sphere plus a ring of mass around its equatorial plane due to its rotation:

Q

Ez

R

l

z

The gravitational potential (or electric potential per unit charge) of the oblate sun about its axis of symmetry is easy to compute. Let us first determine the gravitational potential of the ring of mass on the positive z-axis, and then expand the solution to all space external to the oblate sun. With λ the constant linear mass density of the ring, the potential on the z-axis on a test point, m, is

Uring=-Gml0Mringdm=-Gml02πλdϕ=-2πλGml=-GMml=-GMmz2+R212.

Assume z>R, then

z2+R2-12=1z1+Rz2-12=1z1-12Rz2+1∙32∙4Rz4+…

Then, to first order

Uring=-GMringm1z1-12Rz2=-GMringmz+GMringmR22z3.

The potential obeys Laplace’s equation ∇2V=0 in empty space. Convenient coordinates for our problem are spherical coordinates. See Appendix 1 Chapter 2 for a treatment of Laplace’s equation in these coordinates under azimuthal symmetry. Solutions to Laplace’s equation with azimuthal symmetry are:

Ur,θ=n=0∞anrn+bnrn+1Plcosθ.

Let the test mass lay along some vector, r such that r=z. Then for z>0, we must have an=0, as the corresponding terms tend to infinity as r tends to infinity, contrary to the potential being set to zero at infinity by convention. A review of the binomial expansion above that b2n-1=0:

bor+b1r2+b2r3+b3r4+b4r5+…=-GMmr+GMmR22r3-3GMmR48r5+….

Again, b1=b3=b5=0…. We cut off at b2 in our approximation. Thus,

ϕringr,θ=-GMringmrP0cosθ+GMringmR22r3P1cosθ
=-GMringmr+GMringmR22r3123cos2θ-1,

Q

Ez

R

l

z

r

θ

where θ is shown below:

With r on the z-axis (θ=0), we recover our earlier result:

Uringr,θ=-GMm1z1-12Rr2=-GMringmr+GMringmR22r3

With r on the plane (θ=π2),

Uringr,θ=-GMringm1z1-12Rr2=-GMringmr-GMringmR24r3.

The sphere contributes:

Uspherer,θ=-GMspheremz.

Let Msun=Msphere+Mring. The gravitational field on the equatorial plane of the oblate sun is by:

F=-dUdr= GMsunmr2+3GMringR24r3.

## Orbits about a spherical, then oblate sun – precession of the perihelion

### First things first – two-body barycenter (center of mass) coordinates

r1

r2

r2-r1=r

R

m1

m2

f12

f21

The masses of two bodies orbiting each other may take on any positive value, however, often in practical problems, one mass, i.e., a planet, is much smaller than the other mass, say a star. In the figure below, m1 is a smaller mass relative to the second mass, m2.

If R  is the center of mass (the barycenter) of n bodies, the following equation must be satisfied:

i-1nmiri-R=0.

For two bodies, this means: m1r1-R+m2r2-R=0, or R=m1r1+m2r2m1+m2. The two masses orbit about the barycenter. Let r1' and r2' be the vectors from the barycenter to their respective masses:

r1'=r1-m1r1+m2r2m1+m2  or r1=r1'+R
r2'=r2-m1r1+m2r2m1+m2  or r2=r2'+R

Algebra shows that:

r1'=-m2m1+m2r
r2'=m1m1+m2r.

In constructing a Lagrangian, the total kinetic energy is the energy due to motion of the center of mass, and the motion of the two masses, m1 and m2 about the center of mass:

T=12m1+m2R2+12m1r12+12m2r22=12m1+m2R2+T'.

Algebra shows that T'=12m1+m2R2=12m1m2m1+m2r2. Where r=r2-r1. We defined the reduced by:

μ=12m1m2m1+m2.

The two-body problem of two masses orbiting about their barycenter may be replaced by one fictious particle of mass μ. For the earth Sun system, being respectively, m1 and m2, μ≈m1. The sun effectively sits still while the Earth orbits it. The barycenter is practically the center of mass of the Sun.

### Conservation of angular momentum

The potential function for a perfect sphere, or oblate sphere, does not contain the angle variable, θ. According to Noether’s theorem, angular moment L is conserved. We can reaffirm this by deriving the Euler-Lagrange equation for θ from the Lagrangian:

L=12μr2+r2θ2-Ur.

The Euler-Lagrange equation for θ is:

ddt∂L∂θ-∂L∂θ=μr2dθdt=0.

Thus, μr2θ=L, the constant (conserved) angular momentum. Since L=r×p, its conservation implies that r is always in a plane whose normal is parallel to L. The orbits are planar.

### Orbit equations for U(r) potentials

Note that we have not yet specified the potential function U(r) other than it depends only on r.

ddt∂L∂r-∂L∂r=μr-μrθ2+dUrdr=0.

We get, removing θ,

μr=L2μr3-dUrdr.

#### Differential equation method

μr=-ddr L22μr2-dUrdr
=-ddrL22μr2+Ur.

Multiply both sides by r:

LHS: μrr=ddt12μr2=RHS: -ddrL22μr2+Urdrdt=-ddt L22μr2+Ur=-ddtVeffr,

where Veffr=L22μr2+Ur. Thus

ddt12μr2+Veffr=0.

This means that: 12μr2+Veffr=E=constant. Thus,

r=2μE-Veff(r).

When r=0, we have either one root, corresponding to a circular orbit, or two roots, corresponding to a minimum radius (perigee) and maximum radius (apogee). The two Euler-Lagrange differential equations for the radial and angular coordinate are:

r=drdt=±2μE-Veffr
θ=dθdt=Lμr2.

#### Integral method

These two equations may be used to eliminate time leading to an integral expression for θ in terms of r. Divide the second equation by the first equation and separate variables to obtain:

θr=r0rLμr22μE-Veffrdr+θ0=r0rdrr22μEL2-2μUrL2-1/r2+θ0.

With u=1r, we get

θr=θ0-r0rdu2μEL2-2μU1uL2-u2.

Grab a computer and it’s off to the races.

### Differential equation method continued

Let u=1r. Then du=-1r2drdθ.

dθdr=L2μE-Veffr⟼

dudθ=-1r2μr2L 2μE-Veffr=-μL 2μE-Veffr,

and

d2udθ2=-μL -2μdVeffrdrdrdθ22μE-Veffr=1Ldrdθ2μE-VeffrdVeffrdr
=1LdVeffrdr2μE-Veffrdrdθ=1LdVeffrdr2μE-Veffrμr2L2μE-Veffr
=μr2L2dVeffrdr.

#### Kepler Veffr problem

With k=GMsunmplanet,

Veffr=L22μr2-kr.

d2udθ2=μr2L2L2μr3+kr2=-1r+μkL2.

With u=1r,

d2udθ2+u=1α,

where 1αKepler=μkL2. Now let u=u-1α. The solution is: u=Acosθ-θ0. Without loss of generality, let θ-θ0=π2. When this is true,

1r=u=1α.

From the general solution, it follows that:

dudθ=-Asinθ+θ0=-μL 2μE-Veffr.

Again, let θ+θ0=π2.

A=μL 2μE-Veffr
=μL 2μE-L22μα2+kα=μL 2μE-L22μμ2k2L4+μk2L2
=μL 2μE+μk22L2=2μL2μk22L22EL2μk2+1
=μkL22EL2μk2+1.
Let

e=2EL2μk2+1=2EkL2μk+1=2αEk+1.

Then, with u=u+1α,

1r=μkL2+μkL22EL2μk2+1cosθ+θ0=1α1+ecosθ-θ0.

##### Orbit types

e=0,                 circular
0<e<1,  elliptic
e=1,                 parabolic
e>1,                 hyperbolic

### Asides – radial power law potentials

The asides below deal with different potential functions, e.g. the potential function for a perfect sphere, for an oblate sphere, and many more; see Appendix 2 Chapter 2, on central forces, extends the conversation beyond the two asides below.

#### Power laws and quadrature by circular functions – Goldstein 1980

We are free to explore power law potentials: Ur=arn+1 such that

θr=θ0-r0rdu2μEL2-2μaL2u-n-1-u2.

We know that the integration can be carried out in terms of circular functions (sines and cosines) if the radical has the form αu2+βu+γ, or, equivalently, this means the restriction -n-1=0, 1, 2. The case n=1, the linear case, can be seen to fit the form with the substitution: u2=x implying

du=dx2x.

#### Power laws and quadrature by elliptic functions – Goldstein 1980

An elliptic integral is defined by

R(x,ω)dx

where R is any rational function of x and ω, and ω is defined by ω=αx4+βx3+γx2+δx+η.

Of course, α and β cannot be simultaneously zero, else the integral is solvable in terms of circular functions. Refer to Whittaker and Watson, Modern Analysis, 4th ed., p. 512 to show that elliptical integrals may be reduced to forms involving circular functions and the Legendre elliptic integrals of the first, second, and third kind. Please refer to Herbert Goldstein, Classical Mechanics, 2nd ed., p. 89-90, making the case for n=-4, -5 and -n-1+2=0, 1, 2, 3, 4, n=-5, -7, and n=5, 3, 0, -4, -5, -7. All other cases require numerical integration (quadrature).

## Precessing orbits about the oblate sun

Recall the ring potential from the first section. It follows that

Veffr=L22μr2-GMsunmplanetr-GMringmplanetR24r3

where Msun is the total mass of the sun: sphere + ring, and mring is only the mass of the oblate portion of the sun, e.g., below is an oblate sphere. Of course, since the Earth is spinning as well, it too is an oblate sphere with a 21 km bulge; we will ignore this small “quadrupole” correction.

The r-3 potential term is the quadrupole moment of the gravitational potential of a mass deformed by its rotation in a spherical multipole expansion with azimuthal symmetry (see section on lumpy Earth below). (The generalized multipole expansion does not assume azimuthal symmetry.) As we will see below, the perturbation effect of the oblate sun explained some of the observed precession of the perihelion of Mercury, but not enough. General relativity accounted for the rest. Our equation of motion now becomes:

d2udθ2=μr2L2dVeffrdr=μr2L2ddrL22μr2-GMsunmplanetr-GMringmplanetR24r3
=μr2L2-L2μr3+kr2-3kMringR24Msunr4
=-1r+μkL2+3μkMringR24L2Msunr2
=-u+1α+βαu2,

where

1α=μkL2 and

β=34MringMsunR2.

### Precessing orbits due oblateness – regular perturbation

Recall that for the unperturbed (non-oblate sun), our differential equation for the Kepler problem was

d2udθ2+u=1α,

with solution uθ=1α1+ecosθ-θ0. We could have written

αd2udθ2+αu=1  or d2αudθ2+αu=1.

Let ω0=αu so that our differential equation becomes:

d2ω0dθ2+ω0=1

with solution ω0θ=1+ecosθ-θ0. For the oblate sun, our differential equation was:

d2udθ2+u=1α+βαu2,

or

d2αudθ2+αu=1+βu2

Let ω=αu, and let ϵ=β. We get

d2ωdθ2+ω=1+ϵω2.

Assume the solution to oblate sun to first order in perturbation is of the form:

ωθ=ω0θ+ϵω1θ+Oϵ2.

Substitute our solution in the above differential equation:

ω0θ+ϵω1θ+ω0θ+ϵω1θ=1+ϵω0θ+ϵω1θ+Oϵ22.

The zeroeth terms in ϵ give: ω0θ+ω0θ=1 (our Kepler problem with the above solution). The terms that are first order in ϵ give (using cos2θ=121+cos2θ and letting ϕ=θ-θ0)

ω1ϕ+ω1ϕ=ω02ϕ=1+ecosϕ2=1+e22+e22cos2ϕ+2ecosϕ.

The last term of the particular solution is unbounded, growing in time with angle, θ. Applying the ordinary differential equation method of undetermined coefficients, we guess:
ω1θ=A+Bϕsinϕ +Ccos2ϕ.

Then

ω1ϕ=2Bcos2ϕ-Bϕsinϕ-4Ccos2ϕ.

So

ω1ϕ+ω1ϕ=A+2Bcosϕ-3Ccos2ϕ.

Thus

A=1+e22,  B=e,  C=-e26.

Then

ω1ϕ=1+e22+eϕsinϕ-e26cos2ϕ.

The solution to first order (ωϕ=ω0ϕ+ϵω1ϕ) is then

ωϕ=1+ecosϕ+ϵ1+eϕsinϕ+e212-16cos2ϕ
≈1+ecosϕ+ϵeϕsinϕ≈1+1+ecosϕ1-ϵ.

The period is T=2π1-ϵ=2π1+ϵ+Higher Order.  Thus,

precession ≈2πϵ=2π34MringMsunR2

where h is the angular momentum per unit mass: h=Lμ of the orbiting planet.

#### Dimensional check

αuϕ=1+ecosϕ+34MringMsunR21+eϕsinϕ+e212-16cos2ϕ
1rθ-θ0=uθ-θ0=
=μkL21+2αEk+1cosϕ+34MringMsunR21+2αEk+1ϕsinϕ+2αEk+112-16cos2ϕ
1rθ-θ0≈μkL21+2αEk+1cosθ-θ0+34MringmsunR22αEk+1θ-θ0sinθ-θ0.

## Precession of the perihelion of Mercury and General Relativity

For Mercury, the perihelion advances by 4.9×10-7 radians per revolution, which takes 88 days. In a century, the precession is 203 microradians per century, or approximately 42 seconds of arc per century, which was known as early as 1859 by Urbain Le Verrier. Experimentally, the observed mass of the bulge of the sun, about:

MringMsphere=2.0±8.6×10-6,

too small to produce even 10% of the observed precession of the perihelion of Mercury. The answer came from general relativity:

2πϵ=6πG2Msun2μ2c2L2=4.8×10-7~199microdianscentury.

This comes from the general relativistic equation of motion (Mohana 2020):

d2udϕ2+u=GMsunmplanetμL2+3GMsunc2u2.

The theory of perturbation is rich, following many paths, e.g., ωθ=ω0θ+ϵω1θ+ϵ2ω2θ+…

## Bertrand’s theorem on closed orbits – Goldstein 1980

Recall: energy is conserved: 12μr2+Veffr=12μr2+μrθ2+U(r)=E, with effective potential Veffr=L22μr2+Ur. The radial velocity is therefore given by r=2mE-Veffr.

For a bounded orbit this condition occurs at either one or two radii as seen in an energy diagram. If there is some radius, r0, such that E=Vr0, the bounded orbit is circular. That is r=0.

As seen in the figure above, non-circular orbits having a minimal and maximal radius, rmin and rmax, respectively, may be closed (elliptical orbits), but some may precess in such a way as to not be closed (see figure below of r=r(θ)). For a circular orbit, rθ=constant. For energies at or above Ehyperbola an inbound particle is repelled by the repulsive centrifugal barrier.

For an attractive potential proportional to the inverse cube of the radial distance: Vr=-ar3, the energy diagram below shows that the effective potential has a maximum.

If the initial radius is less than r1, the particle is trapped (cannot tunnel out as would be the quantum mechanical case) and will eventually pass through the center of force. The initial radius cannot be in the interval r1, r2. If the initial radius is greater than r2, the motion of the particle passes through the center of the force and is unbounded.

Returning to circular orbits, requirement that Veff(r) have an extremum is equivalent to the vanishing of f'ro for some ro. Given the above effective potential for some radial potential, the effective force is

feffr=dVeffrdr=L2mr3+dUrdr=L2mr3+fr.

At fro=-L2mr03, feffro=0, and feff(r) (the force) is the slope of the potential, zero at ro.

## Stability or circular orbits – Goldstein 1980

The stability of a circular orbit (to perturbations) is determined by the sign of the second derivative of Veff(r) at the radius of the circular orbit, r0. An orbit is stable for a positive second derivative (Veff'(r) concave up) and unstable for Veff'(r) concave down. A stable orbit therefore occurs if

∂2Veff∂rr=r0=-∂f∂rr=r0+3L2mr04>0.

That is, a circular orbit is stable if:

∂f∂rr=r0<-3L2mr04,

or

dlnfdlnrr=r0<-3.

If the force behaves as a power law of r in the vicinity of the circular radius r0, namely

fr=-krn+1,

the first stability equation becomes:

-n+1kr0n+2<-3kr0n+2,

or n<2. Clearly, the potential Vr=-ar3 corresponding to the force fr=-3ar4 is unstable about the circular orbit at the maximum of the effective potential.

### Proof of Bertrand’s Theorem

See Goldstein 1980 Appendix A. Conclusions: fr=-kr2 (Kepler) and fr=-kr (Hook’s law) are the only two possible force laws whose circular orbits are stable to large perturbations that keep the orbits closed. For small perturbations still resulting in closed orbits, the situation is more relaxed:

fr=-kr3-β2,  β rational.

## Perturbation (such as from asteroid strikes) and stability of orbits

No orbital calculation can include all forces that act on an orbiting body. Some are unpredictable, such as the burst of solar wind or a coronal mass ejection, a chance encounter with an asteroid, and so forth. Many forces of small and short duration will perturb orbits. We ask: are (nearly) circular orbits stable? Stability should depend on every small displacement leading to a restoring force.

### Example regarding the stability of circular orbits

Consider a small point mass orbiting in circles inside an inverted, frictionless cone with vertical z-axis:

Imagine a fly bounces off the particle such that R→R+x. Is the circular orbit stable? Does the perturbation lead to a restoring force? Or does the mass fall to the bottom or fly over the top? Let’s construct the equations of motion from the Lagrangian.

We have

Rz=ξsinαξcosα  →  z=Rcotα

Thus our kinetic energy is given by:

12mv2=12mR2+R2θ2+z2=12mR2+R2θ2+R2cot2α=12mR2csc2α+R2θ2.

The potential energy is given by mgr cotα. Hence our Lagrangian is

L=12mR2csc2α+R2θ2-mgRcotα.

The Euler-Lagrange equation for the angular coordinate, θ, we are reminded that angular momentum is conserved with l=mR2θ. The radial Euler-Lagrange equation yields

R-Rθ2sin2α+gsinαcosα=0.

For a circular (horizontal) orbit, R=0, Rθ2sin2α=gsinαcosα. For some initial conditions with z=R=constant and θ2=gRcotα.

#### Stability to displacement perturbation

Assume something strikes the orbiting body, displacing its radius to R→R+x – Boom! Do we fly away from our parent star if xR≪1? Consider the following for our particle orbiting in an inverted cone.

Rθ2=Rl2m2R4=l2m2R3=→l2m2R+x3=l2m2R3 11+xR3.

A truncated binomial expansion shows that

Rθ2=l2m2R31-3xR .

Since we assumed a circular orbit, R=0, our radial differential equation becomes

0+x-R+xθ2sin2α+gsinαcosα=0,

or

x-xθ2sin2α-Rθ2sin2α+gsinαcosα=0.

With Rθ2≈l2m2R31-3xR, and with conservation of angular momentum, we have

x+l2sin2αm2R4x-1-3xRl2m2R3sin2α+gsinαcosα=0.

or

x+2l2sin2αm2R4x-l2m2R3sin2α+gsinαcosα=0.

x+2l2sin2αm2R4x-Rθ2sin2α+gsinαcosα=0.

x+2l2sin2αm2R4x=0.

This is the equation of a harmonic oscillator in the form of Hook’s restoring force law, and with frequency ω=2lmR2sinα, a stable, oscillating orbit over and under our initial height, z.

## A lumpy Earth and the bumpy rides for satellites – spherical harmonics

A sufficiently irregular body cannot be treated as a point source particle. As analyzed above assuming azimuthal symmetry, spinning bodies are oblate. Near the earth, say in low earth orbing (LEO), the planet is lumpy (not azimuthally symmetric), with mountain ranges above and below water. Azimuthal symmetry allows for describing the gravitational potential in terms of spherical harmonics (of the Legendre equation). Non-azimuthal symmetry requires tesseral harmonics (today also called spherical harmonics, but for the full Laplace’s equation in spherical coordinates.)

P

R

r

r

θ

dτ

Case 1 - Azimuthal symmetry – spherical harmonics

Assuming azimuthal symmetry, let’s derive this treatment for, say, m=msatellite and M=Mearth.

UPm=-Gρrdτ=-GM2π1rdθ.

By the law of cosines: r2=R2+r2-2rRcosθ=R21+r2R2-2rRcosθ. Thus, r=1+ϵ, where ϵ=rRrR-2cosθ. If R is large with respect to the dimensions of the mass (or charge blob), than ϵ is small and we be expand the radical using a binomial expansion (or equivalent Taylor expansion) such that

1r=1R1+ϵ-12=1R1-12ϵ+38ϵ2-516ϵ3+…

That is

1r=1R1-12rRrR-2cosθ+38rR2rR-2cosθ2-516rR3rR-2cosθ3+…
=1R1+rRcosθ+rR232cos2θ-12+rR352cos3θ-32cosθ+…
=1Rn=0∞rRnPncosθ.

The Pn are the Legendre polynomials. Expressed in terms of a multipole expansion,

UPm=-GMn=0∞1Rn+1rnPncosθρdτ.

The multipole expansion technique provides a way of approximating the potential at points far away from a charge distribution: monopole + dipole + quadrupole +… moments +…. A perfectly symmetric sphere of mass, M, is described by only the zeroeth order (or monopole) term of the series, P0, leading to Fr=-dUdr~-1/r2. The first term (or dipole moment), P1, produces zero as integrating cosθ from 0 to 2π results in zero; all terms with odd powers of cosine, in fact, drop out for the same reason. Oblateness is picked up by the second (quadrupole) term, P2. To obtain the quadrupole moment properly for the oblate sun, we would have to integrate the mass term over volume the oblate sphere, but, as you know, there is an approximation that eases this calculation, namely, treating an oblate sphere as a sphere with a ring of mass wrapped around it (see the first section).

#### The monopole moment

A perfectly symmetric sphere of mass M is described by the zeroeth order (or monopole) term of the series leading to a gravitational potential per unit mass, m, URm=GMR such FR=-dURdR=GMmR2, Newtons law of universal gravitation between point particles. All other terms are zero for a perfectly spherical body. (To avoid confusion, we have been using r mostly, not R.)

#### The dipole moment – for electrostatics, not gravitostatics

The first order term (or dipole moment), P1, drops out for a massive body of any shape as integrating cosθ from 0 to 2π results in zero; all terms with odd powers of cosine, in fact, drop out for the same reason. These cancellations do not necessarily hold in electrostatic problems. If, for example, we place a positive electric charge above a metal sphere hung by a string, the free electrons of the metal will rearrange themselves to produce an electric dipole:

Negative charges concentrate at over the top half leaving a net positive charge over the bottom half. This does not happen for matter. If antimatter antigravitates, gravitational dipoles could exist. The consensus, however, is that gravity acts indistinguishably on matter and antimatter. That is, despite the fact that antimatter has the opposite electrical charge of its matter counterpart, as far as gravity goes, matter and antimatter are the same (have the same attractive, positive charge).

See Appendix 3 Chapter 2 regarding gravitational monopole and dipole radiation in classical gravity and general relativity.)

#### The quadrupole moment – oblate spheres

The gravitational potential of an oblate sphere is given by the sum of a monopole and quadrupole moment – which we constructed with a clever cheat: summing the monopole potential (Pn=0=1) of a spherical sun with a ring about the sun’s equator (quadrupole moment, Pn=2=123cos2θ-1).

Ur=-GMsunmplanetr-GMringmplanetR24r3.

That is (where the variable r above is the variable R in the multipole series):

-1Rr0P0(cosθ)ρdτ=-GmplanetRρdτ=-GMsunmplanetR

and

1R3r2P2(cosθ)ρdτ=GMringmplanetRring24R3

No higher order terms are required to describe the potential of an oblate sphere.

#### Higher order multipole moments

The more distorted the body (keeping azimuthal symmetry about the z-axis), the more terms are multipole -terms are required to describe its gravitational potential, e.g.

## Full spherical harmonics (tesseral harmonics)

Spinning rocky objects such as Earth are more than irregular than oblate. They tend to be covered with mountain ranges that break azimuthal symmetry. The spherical harmonics (associated with the Legendre differential equation) are insufficient, and must be generalized to the spherical harmonics (for the associated Legendre differential equation) – historically called tesseral harmonics (Jackson 1975):

Ylmθ,ϕ=2l+14πl-m!l+m!Plmcosθeimϕ.

The potential is then given by

Φr,θ,ϕ=l=0∞m=-llAlmrl+Blmr-l+1Ylmθ,ϕ.

The Alm terms are for expansions inside bodies, say hollow metal blobs for electrostatic potentials. Outside a body, these terms are set to zero since the rl are unbounded as r→∞. Conventionally, we set potentials to zero at infinity. Outside bodies, we use the Blmr-l+1 terms. Note that perturbations from perfect sphericity, drop to zero increasingly rapidly with distance from the gravitating mass (or charged blob), dropping increasingly faster the higher the order of the perturbation term: 1r, 1r3, 1r5….

Satellites in low Earth orbit feel more of Earth’s perturbations than satellites higher up, say in geosynchronous orbits, and hence require more frequent orbital corrections (and more fuel) than satellites at higher orbits.

### Addition theorem for spherical harmonics – Jackson 1975

A remarkable result is the addition theorem for spherical harmonics, which states that for two coordinate vectors, x and x', with spherical coordinates r,θ,ϕ and r',θ',ϕ', and angle γ between them, a Legendre polynomial of order l in the angle γ can be expressed in terms of the products of the spherical harmonics in angles θ,ϕ and θ',ϕ':

Plcosγ=4π2l+1m=-llYlm*θ',ϕ'Ylmθ,ϕ

where cosγ=cosθcosθ'+sinθsinθ'cosϕ-ϕ'.

## Orthogonality of special functions

The special functions of mathematical physics, including sines and cosines, Legendre polynomials, associated Legendre polynomials, Hermite polynomials, Laguerre polynomials, etc., are orthogonal over some domain, e.g.,

02πdϕ0πsinθdθ Ylm*θ',ϕ'Ylmθ,ϕ=δl'lδm'm.

## Gravitational boosts (assists)

From the planet’s frame, an inbound probe gains speed as it falls towards it, and then loses this speed as it leaves. From the point of view of the sun, the vector of the probe is changed, increasing its velocity with respect to the sun – see figure below from Wikipedia. Gravitational boosting is how the Voyagers escaped the solar system.

We may treat this as a simple elastic collision problem. Energy and momentum are conserved. That is

12m1u12+12m2u22=12m1v12+12m2v22

m1u1+m2u2=m1v1+m2v2.

Given u1 and u2,

v1=m1-m2m1+m2u1+2m2m1+m2u2
v2=2m1m1+m2u1+m2-m1m1+m2u2

If m1 is the mass of the probe,  m1≪m2. Hence

v1≈-u1+2u2
v2≈u2.

## Notes on the Schrödinger equation

In the first quarter of the twentieth century, physicists figured how to turn classical Hamiltonians (transformed Lagrangians) into quantum mechanical Hamiltonians, and classical equations of motion into quantized equations of motion. One way, natural to these notes is mapping classical momentum and energy to differential operators, e.g.,

pq→-iℏ∂∂q

where q is a canonical momentum coordinate, e.g., p22m for the energy in the x or y or z  or θ or ϕ or other canonical coordinates. Thus, along the x-axis

px22m=-iℏ22m∂2∂x2 .

The energy, E, is replaced by iℏ∂∂t. Construct a classical Lagrangian and thusly convert it to a quantum mechanical equation (the appropriate Schrödinger equation). For the classical harmonic oscillator in one dimension,

H=p22m+12kx2→-ℏ22m∂2ψ∂x2+12kx2ψ=Eψ.

We have the Hamiltonian for a quantized one-dimensional harmonic oscillator, a great model for vibrational spectroscopy.

Switching to an electron orbiting a proton, a hydrogen atom, thanks to the inverse square Coulombic law of attraction between opposite charges, we begin with the classical Hamiltonian in spherical coordinates:

H=-ℏ22μ ∇2-Ze24πϵ0r=E.

After the above transformations, we have Schrödinger equation for the hydrogen atom:

1r2∂∂rr2∂ψ∂r +1r21sinθ∂∂θ sinθ∂ψ∂θ +1r21sin2θ∂2ψ∂ϕ2+2μℏ2 E+Z4πϵ0rψ=0,

solved using separation of variables: ψ=RrΘθΦ(ϕ). Solutions involve the associated Laguerre polynomials.

As opposed to building the potential of a lumpy mass, e.g., Earth, the orbiting electron can only be in one of the orbitals, making discrete jumps up or down in energy as it absorbs or emits photons. See orbitals in the figure below.

## Appendix 1 Chapter 2 - Laplace’s equation in spherical coordinates (for electrostatics or ‘gravitostatics’) – Griffiths 1981

In electrostatics, in regions where there is charge, Gauss’ law states ∇∙E=ρϵ0. Moreover (in electrostatics) ∇×E=0. Since E=-∇V, where V, is the potential function, we have Poison’s equation. What applies in electrostatics applies to gravitostatics as well. Poison’s equation:

∇2V=-ρϵ0.

In electrostatics, where there is no charge, we have Laplace’s equation: ∇2V=0. In spherical coordinates, Laplace’s equation is:

∇2V=1r2∂∂rr2∂V∂r+1r2sinθ∂∂θsinθ∂V∂r+1r2sin2θ ∂2V∂ϕ2=0.

With azimuthal symmetry, Vr,θ,ϕ=Vr,θ; V is independent of ϕ. Laplace’s equation reduces to

∂∂rr2∂V∂r+1sinθ∂∂θsinθ∂V∂r=0.

Separation of variables (with Vr,θ=RrΘθ) leads to

1R(r)ddrr2dRrdr+1ΘθsinθddθsinθdΘθdθ=0.

Since the first term depends only on r and the second term only on θ, each must equal a constant. The ‘constant’ that works, as mathematicians determined, is l(l+1) where l=0, 1, 2….

1R(r)ddrr2dRrdr=l(l+1)

1ΘθsinθddθsinθdΘθdθ=-ll+1.

The general solution of the radial equation

ddrr2dRrdr=ll+1Rr  is:

Rr=Arl+Brl+1.

The angular equation is related to Legendre’s equation:

1sinθddθsinθdΘθdθ=-ll+1Θ(θ)

Solutions are Legendre polynomials: Θθ=Pl(cosθ), where Pl(x) is given by the Rodrigues formula:

Plx=12ll!ddxlx2-1l.

These polynomials are orthogonal over [-1, 1]:

-11PlxPkx=0  if k≠l.

Consequently, solutions in three dimensional space with azimuthal symmetry are:

Vr,θ=n=0∞Arn+Brn+1Plcosθ.

### Aside

The Rodrigues formula provides one solution to our second order differential equation. We need another solution, and/or to show that the missing second solution is unphysical. This aside treats this issue with the substitution x=cosθ. This x is not a Cartesian coordinate. Then let Θθ=Zx to get

1-x2d2Zdx2-2xdZdx+ll+1Z=0.

This above is Legendre’s equation. For the general problem where V=V(r,θ,ϕ), we would have arrived at Legendre’s associated equation:

1-x2d2Zdx2-2xdZdx+ll+1-m21-x2Z=0.

The solutions are the associated Legendre polynomials (Whittaker and Watson 1927).

In quantum mechanics, say the Schrödinger equation model of the hydrogen atom, l(l+1) has the physical interpretation of the square of the total angular momentum of a particle, while m (integral-valued) is the component of angular momentum about the z-axis. Spin, along the z-axis, is half integral-valued, and these cases do not correspond to a differential equation.

When the Legendre equation is solved by power series (method of Frobenius), one finds that if constant=l(l+1), where l is a nonnegative integer, then there is one solution (of the two independent ones) that is a polynomial — the Frobenius series terminates. These are called the Legendre polynomials, Plx. A totally different way of stumbling upon them is to apply the Gram–Schmidt orthogonalization procedure to the sequence of powers, 1, x, x2, …, regarded as functions on the interval [−1, 1] with the usual inner product.

Just as we required solutions in polar coordinates to be bounded at the origin, we must require solutions in spherical coordinates to be bounded at the north and south poles x=±1. It is a verifiable fact that all solutions except the Legendre polynomials behave unacceptably at one or the other of the endpoints. In our problem, therefore, the eigenvalues are the numbers l(l+1), and the Legendre polynomials are the eigenvectors.

The other solutions become relevant in other problems where the region does not contain the whole sphere, e.g., a cone. When our separation constant is l(l+1) so that Pl exists, another linearly independent solution can be found by the method of reduction of order, or the Frobenius theory:

Q0x=12ln1+x1-x,  Q1x=x2ln1+x1-x-1.

## Appendix 2 Chapter 2 - Central forces

A central force is a force (possibly negative) that points from the particle directly towards a fixed point in space, the center, and whose magnitude only depends on the distance of the object to the center. Central forces of attraction include Newton’s law of universal gravitation for point-like masses and Coulomb’s law for point-like charges, with these two exemplifying inverse square law forces. That is, their magnitudes are inversely proportional to the square of the distance separating two point-like bodies.

Why point-like?

In typical central force gravitational problems, e.g., planets orbiting a star or satellites (real or artificial) orbiting a planet, the masses are approximately spherically distributed. According to the shell theorem, which Isaac Newton employed, such spherically symmetric bodies may be treated as if all their masses were concentrated at their centers of mass at the bodies’ origins, that is, as point-like source particles, or simply, point particles.

When deviations from sphericity are significant, as in the case of the oblate sun or lumpy, oblate earth, we have seen that Newton’s and Coulomb’s inverse square laws require the addition of multipole moments in spherical coordinates. Many other coordinate choices are available. Which one to choose is the one that is most suited for the distribution of the mass or charge, e.g., cylindrical, rectangular, etc.

Of course, as we have seen in the asides treated above, there are many other types of central force fields besides inverse square central force fields, including linear combinations of power laws. In a simplified model of interactions between electrically neutral atoms and/or molecules, for example, the Lennard-Jones 12-6 potential description is quite useful,

VLJ=4ϵσr12-σr6.

The central Lennard-Jones force is F=-dVLJdr. For further readings on interatomic potentials see this link.

The correction from general relativity, interestingly enough, has the same form: FGRr=Cr-2+Dr-4, but with different coefficients. This type of central force is solvable by circular functions, as is Fr=Ar-3+Br and Fr=Ar-5 for, in particular, circular orbits passing through the point of attraction.

Wikipedia, in their article on the classical central-force problem, provides six more examples of linear combinations solvable in terms of circular and elliptic functions. Central forces not solvable by known functions, e.g., circular and/or elliptic functions, are generally solvable by quadrature, that is, by numerical integration methods.

Note that forces of attraction need not be central forces. Two masses, m1 and m2 acting upon themselves with attractive forces f12 and f21 such that f12=f=-f21 to satisfy Newton’s third law, will also orbit the barycenter.

## Appendix 3 Chapter 2 – No gravitational dipole (or monopole) radiation – except…

The consensus on antimatter is that gravity acts indistinguishably upon it as it acts upon its matter counterpart. Despite the fact that antimatter has the opposite electrical charge of its matter version, as far as gravity goes, it appears that matter and its antimatter counterpart have the same, self-attracting ‘gravitational charge’. Experiment, of course, will have the final word. In 2018, CERN began testing if antimatter antigravitates, or perhaps if it falls at a different rate than matter.

The simplest electric dipole moment is formed when we place a positive charge, q, a distance, d, from an opposite charge -q. The dipole moment is, p=qd, where d is the vector of length d between the charges point from the negative charge to the positive charge. For a distribution of positive and negative charges, the resultant dipole moment would be derived through vector addition over all dipole pairs. The second figure in Appendix 1, depicts a dipole moment created by placing a positive point charge above a metal sphere. If matter and antimatter are indistinguishable to gravity, the contribution to the gravitational potential of a blob of mass from n=1 (the dipole moment term) is zero:

1R2rP1cosθρdτ=0.

Oscillating electric dipoles lead to electromagnetic dipole radiation, but as gravitational dipoles do not exist, neither does classical gravitational dipole radiation.

In general relativity, in the linearized Einstein field equations for the perturbed metric, hμν, gravitational waves are propagated by helicity 2 in the presence of a stress-energy pseudo-tensor, τμν according to

Xhμν=-16πτμν,  ∂μhμν=0,  ∂μτμν=0.

Since τμν is conserved, there are no monopole or dipole moment contributions to gravitational waves. However, it has been shown that gravitationally unbound mass quadrupoles, despite having zero mass dipole moment, nevertheless produce dipole perturbations of gravitational fields that do not completely destructively interfere as they propagate to the radiation zone, that is, to distances much greater than the perturbation wavelengths.

The source of a gravitational field, the active gravitational mass density, sometimes referred to as Whittaker’s mass density, is ρgravity=ρ+3pc2,  where p is pressure in the case of an ideal fluid. Whittaker’s mass is not conserved. Changes to it can propagate as monopole gravitational waves. Such waves can be generated, for example, from  astrophysical sources with varying gravitational mass such as relativistic fireballs driving gamma-ray bursts.

## References Chapter 2

R. Baierlein, Newtonian Dynamics, McGraw-Hill Book Company, 1983

D. J. Griffiths, Introduction to Electrodynamics, Prentice-Hall Inc., 1981

H. Goldstein, Classical Mechanics, 2nd ed., Columbia University, 1980

J. D. Jackson, Classical Electrodynamics, 2nd ed., Wiley, 1975

I. Mohana, The General Relativistic Precession of Mercury’s Perihelion and the Corrections to Newton’s Gravitational Potential and Force, ResearchGate, 2020

F. Y. M. Wan, Mathematical Models and Their Analysis, Harper & Row, 1989

E. T. Whitaker and G. N. Watson, A Course on Modern Analysis, Cambridge University Press, 1927